|  | 506/507/508 - HA389427 |  |
| --- | --- | --- |
|  | Chapter | 7 |
|  | Technical Specification |  |
|  | Contents Page |  |

- Input  Supply

- Speed Control

- Current (Torque) Control

- Adjustment Range

- Environmental Requirements

- EMC Technical Ratings

- Immunity

- Emissions

- Electrical  Ratings

- Output Ratings with typical armature voltage, Va, 80/90V (160/180V)

- Adjustment Ranges

- Product Code

# Technical Specifications

| ### Input Supply |  |  |  |
| --- | --- | --- | --- |
| Parameter | Symbol | Value | Units |
| Frequency Voltage | Vs | 50/60 ± 10% 110/120 ±10% or 220/240 ± 10% (Earth (TN) or non-earth (IT) referenced) | Hz V |

| ### Speed Control (1) Range selected by built in switches. (2) With IR comp setting optimised. (3) Consideration must be given to the motor at it may overheat at low speed. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Parameter | Symbol | Value |  | Units |  |
| Stall detect time Type of controller |  | 15 maximum 20 typical Variable proportional plus integral |  |  | s |
| Feedback method (1) |  | Va | Tacho |  |  |
| 0-100% load regulation Max. torque speed range |  | 2(2) (typically) 20:1 | 0.1 (typically) 100:1 (3) |  | % |

| ### Current (Torque) Control (1) Range selected by built in switches. (2) Suitable for armature time constants not less than 5ms. (3) All control terminals are at a potential of the peak of the input supply with respect to earth (ground) . All cables should be rated for this voltage. |  |  |  |
| --- | --- | --- | --- |
| Parameter | Symbol | Value | Units |
| Overspeed limiting (1) Type of controller (2) Feedback method Linearity |  | Standard (1) Fixed proportional plus integral Non-isolated shunt (3) 2 | % |

| ### Adjustment Range |  |  |  |
| --- | --- | --- | --- |
| Parameter | Symbol | Value | Units |
| Armature loss comp. Maximum speed Minimum speed Ramp up time Ramp down time | IR comp N max N min Ramp Ramp | 0 - 25% of Va 100% ± 25% 0 - 25% of N max 1 - 15 (minimum) 1 - 15 (minimum) | % s s |

| ### Environmental Requirements |  |
| --- | --- |
| Operating Temperature | 0 to +40oC (Derate 1.5% / Degree above 40oC) |
| Max. Operating Temperature | 50oC |
| Humidity | 85% R.H. at 40øC (non condensing) |
| Altitude | Above 1000m derate at 1% / 100m |
| Atmosphere | Non-flammable, non-corrosive and dust free |
| Storage Temperature Range | -25oC to +55oC |
| Transport Temperature Range | -25oC to +55oC |
| Enclosure | IP2X suitable for cubicle mounting |
| Installation Category | Overvoltage Category III. |
| Pollution | Pollution Degree 2. |

| ### EMC Technical Ratings #### Immunity |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Port | Phenomenon | Test Standard | Level | Criterion | Generic Standard |
| Enclosure Port | ESD RF Field | BS EN61000-4-2(1995) | 8kV AD 10V/m,1kHz AM | Self Recovery No Change | EN50082-1 (1992), |
| Power Ports | Fast Transient Burst | BS EN61000-4-4(1995) | 2kV | Self Recovery | and EN50082 |
| Signal & Control | Fast Transient Burst | BS EN61000-4-4(1995) | 2kV | Self Recovery | -2 (1995) |
| Power Interfaces | Fast Transient Burst | BS EN61000-4-4(1995) | 2kV | Self Recovery |  |

| #### Emissions These levels of performance are achieved when installed as specified with the recommended Supply Filter. * Achieved with up to 50m of motor cable. |  |  |  |  |
| --- | --- | --- | --- | --- |
| Port | Phenomenon | Test Standard | Level | Generic Standard |
| Enclosure Port | Radiated | EN55011 | Class B | EN50081-1 (1992) |
| Power Port | Conducted | EN55011 | Class B* | EN50081-2 (1994) |

## Electrical  Ratings

|  | ### Output Ratings with typical armature voltage, Va, 80/90V (160/180V) (1) Range selected by built in switches. (2) When the controller is energised the dc motor shunt field is permanently energised. It is the responsibility of the installer to ensure that this is not detrimental to the long term reliability of the dc motor. |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Parameter | 506 |  |  | 507 |  | 508 |  |  |
|  | Maximum Armature Output Current (IA) (1) | 1.5A | 3A | 3A |  | 6A |  | 6A | 12A |
|  | Typical Motor Horsepower (Hp) 90V Armature (110/120V AC) 180V Armature (220/240V AC) | 1/10 1/6 | 1/6 1/3 | 1/6 1/3 |  | 1/3 3/4 |  | 1/3 3/4 | 3/4 2 |
|  | Typical Motor Power (kW) 90V Armature (110/120V AC) 180V Armature (220/240V AC) | 0.07 0.125 | 0.125 0.25 | 0.125 0.25 |  | 0.25 0.55 |  | 0.25 0.55 | 0.55 1.5 |
|  | Controller loss (W) | 5 | 10 | 10 |  | 20 |  | 20 | 40 |
|  | Maximum AC Supply Current (ls,A rms) | 3 | 4.5 | 6 |  | 9 |  | 12 | 18 |
|  | Maximum l2t for fusing (A2s) | 36 | 36 | 80 |  | 80 |  | 365 | 365 |
|  | Maximum form factor (ls/la) | 2 | 1.5 | 2 |  | 1.5 |  | 2 | 1.5 |
| Maximum Field Current (2) (If, A dc) |  | 2 |  |  |  |  |  |  |  |
| Field Voltage (0.9 x Vs) V dc 110/120V AC 220/240V AC |  | V 90-100 180-200 |  |  |  |  |  |  |  |

| ### Adjustment Ranges * Range selected by built in switches. |  |  |  |
| --- | --- | --- | --- |
| Parameter | 506 | 507 | 508 |
| Maximum current * (l max, A) | 0.25 - 3 | 0.5 - 6 | 1 - 12 |

| ### Product Code |  |  |  |
| --- | --- | --- | --- |
| Block | Product | Code | Feature |
| 1 | Basic Product | 506 | 3 Amp |
|  |  | 507 | 6 Amp |
|  |  | 508 | 12 Amp |
| 2 | Livery | 00 | Standard |
|  |  | 01 to 99 | Customer |
| 3 | Cover | 20 | IP20 Cover |
| 4 | Special Options | 00 | Standard |
|  |  | 01 to 99 | Documented Special Options |