|  | 584SV Series Frequency Inverter - HA463660 |  |
| --- | --- | --- |
|  | Chapter | 1 |
|  | Getting Started |  |
|  | Contents Page |  |

- Introduction

- Standard Equipment

- Optional Equipment

- About this Manual

- How the Manual is Organised

# Getting Started

## Introduction

The Types 8, 9 and 10 form the High Power AC (HPAC) range of the 584SV Inverters. In common with the lower power Inverters (Types 4, 5, 6 & 7), the dual mode feature (constant or quadratic torque) provides a cost effective solution to general industrial applications, as well as the control of pumps and fans.

The optional internal RFI filters offer enhanced EMC compliance without the need for additional external components (where fitted).

Option cards can be fitted inside the Inverter to give serial communications, closed loop speed control, and the factory-fitted dynamic braking functions.

| 584SV Type | Nominal Power Constant Torque 380VAC | Nominal Power Constant Torque 460VAC | Nominal Power Quadratic Torque 380VAC | Nominal Power Quadratic Torque 460VAC |
| --- | --- | --- | --- | --- |
| 8 | 90 - 132kW | 150 - 200Hp | 110 - 150kW | 150 - 250Hp |
| 9 | 160 - 200kW | 250 - 300Hp | 200 - 250kW | 300 - 400Hp |
| 10 | 250 - 280kW | 400 - 450Hp | 300 - 315kW | 450 - 500Hp |

Table 1-1  Summary of Ratings

The models are identified by the product code. Refer to Chapter 2 “An Overview of the Inverter” - Understanding the Product Code.

### Standard Equipment

The following equipment is supplied as standard with each Inverter:

- Exhaust duct and top vent assembly

- Main cooling fan (Type 10 only)

- 5841 Operator Station (inverter-mounted)

- Lifting eyes (4 off)

- Product Manual and Addendum

### Optional Equipment

| Item | Part Number |
| --- | --- |
| 5841 Operator Station (for remote-mounting) A unit providing control, monitoring and programming capabilities Panel Mounting Kit for the 5841 Operator Station | 5841 5842 |
| Braking Resistor A resistor fitted for transient braking operations 36W, 500W 56W, 200W | CZ388396 CZ463068 |
| EMC Installation Guidelines for Modules and Systems A Eurotherm Drives application manual detailing EMC requirements | HA388879 |
| 5843 Technology Option Plug-in field bus communications interface options: - Profibus - Profibus Technology Option Manual - RS485 - RS485 Technology Option Manual - Link - Link Technology Option Manual | AH463470U001 HA463561 AH463469U001 HA463560 AH470016 HA470237 |
| ConfigEd Lite Eurotherm Drives’ Windows-based block programming software | Order by name |
| Dynamic Braking Module A factory-fitted module | Factory fitted option |
| External AC Supply (RFI) Filter | Refer to Chapter 11: “External AC Supply (RFI) Filters and AC Line Chokes” |

## About this Manual

This manual is an Addendum to the 584SV Frequency Inverter Product Manual (Part No. HA463617)

Every Inverter Type 8, 9 and 10 is shipped with this Addendum and the relevant Product Manual listed above. Refer to the Product Manual for details of the following:

- commissioning procedures

- safety information

- control specification

- software configuration

- diagnostics

- fault finding

- application information

### How the Manual is Organised

Text in this Addendum is either in addition to, or is a replacement for equivalent text found in the main Product Manual.

- You will find that chapter numbers and titles are synchronised between this Addendum and the 584SV Product Manual.